| VMnet | Subnet (/29) | Devices Connected | Example IPs |
| --- | --- | --- | --- |
| VMnet2 | 36.0.0.0/29 | Attacker ↔ Kali WAN | kali: 36.0.0.1, Kali: 36.0.0.2 |
| VMnet3 | 136.0.0.0/29 | Windows Internal ↔ SecureGate WAN | kali: 136.0.0.1, SG: 136.0.0.2 |
| VMnet4 | 200.0.0.0/29 | SecureGate LAN ↔ Internal Router | SG: 200.0.0.1, Router: 200.0.0.14 |
| VMnet5 | 222.0.0.0/29 | SecureGate Honeypot ↔ Honeypot | SG: 222.0.0.1, HP: 222.0.0.2 |

Also dhcp is closed here.

1. Configured VMware Virtual Networks

You created the following custom VMnet networks:

| VMnet | Subnet | Mask | Purpose |
| --- | --- | --- | --- |
| VMnet2 | 36.0.0.0/29 | 255.255.255.248 | Attacker ↔ Windows (WAN) |
| VMnet3 | 136.0.0.0/29 | 255.255.255.248 | Windows ↔ SecureGate (WAN) |
| VMnet4 | 200.0.0.0/29 | 255.255.255.248 | SecureGate ↔ Internal Router (LAN) |
| VMnet5 | 222.0.0.0/29 | 255.255.255.248 | SecureGate ↔ Honeypot |

✔ DHCP on VMnet5 (honeypot) was disabled
✔ All networks use static IPs only

2. Mapped Virtual NICs to VMnets

Attacker Kali

- eth0 → VMnet2

kali outer Router VM

- Ethernet2 → VMnet2

- Ethernet0 → VMnet3

SecureGate Firewall (Kali)

- eth0 → VMnet3

- eth1 → VMnet4

- eth2 → VMnet5

Internal Router (Kali)

- eth0 → VMnet4

Server (Kali)

- eth0 → VMnet4

Honeypot (Ubuntu)

- ens33 → VMnet5

3. Assigned Static IPs to All VMs (Only IP Assignment Completed)

Below is exactly what has been done so far.

✅ A. Attacker Kali (VMnet2 — 36.0.0.0/29)

Command used:

sudo ip addr add 36.0.0.2/29 dev eth0

Result:

IP: 36.0.0.2

Mask: 255.255.255.248

✅ B. kali Router VM

External Interface (Ethernet2 → VMnet2)

IP: 36.0.0.1

Mask: 255.255.255.248

Gateway: (not configured yet)

Internal Interface (Ethernet0 → VMnet3)

IP: 136.0.0.1

Mask: 255.255.255.248

Gateway: (not configured yet)

✅ C. SecureGate Firewall (Kali)

As of now, you assigned IPs one-by-one (at different times), but final intended IPs are:

WAN (eth0 → VMnet3)

sudo ip addr add 136.0.0.1/29 dev eth0

LAN (eth1 → VMnet4)

sudo ip addr add 200.0.0.1/29 dev eth1

Honeypot Interface (eth2 → VMnet5)

sudo ip addr add 222.0.0.1/29 dev eth2

✅ D. Internal Router (Kali → VMnet4)

Command:

sudo ip addr add 200.0.0.14/29 dev eth0

✅ E. Server VM (Kali → VMnet4)

Command:

sudo ip addr add 200.0.0.2/29 dev eth0

✅ F. Honeypot Ubuntu (VMnet5)

Command:

sudo ip addr add 222.0.0.2/29 dev ens33

commands on outer (WAN) router:

Perfect — now that your outer router (Kali Router VM) is successfully connected and working, here is a clean, organized write-up of:

✔ Every step you performed

✔ Commands used

✔ Why each step was required

✔ What role it plays in your network topology

You can directly use this in your documentation or project report.

🚀 OUTER ROUTER CONFIGURATION — COMPLETE DOCUMENTATION

Your Outer Router VM (Kali Linux) is acting as a Layer-3 router between:

- External/Attacker Network (36.0.0.0/29)

- SecureGate Network (136.0.0.0/29)

It forwards packets from attackers → SecureGate and from SecureGate → attackers.

1️⃣ Checking available network interfaces

Command:

ifconfig

Purpose:

- To identify interface names → here eth0 and eth1.

- To confirm that no IPs were assigned before configuration.

- To match interfaces with VMnet settings (eth0 = VMnet2, eth1 = VMnet3).

This step ensures you assign the correct IP to the correct NIC.

2️⃣ Assigning IP addresses temporarily

For Attacker-side interface (eth0):

sudo ip addr add 36.0.0.1/29 dev eth0

sudo ip link set eth0 up

For SecureGate-side interface (eth1):

sudo ip addr add 136.0.0.1/29 dev eth1

sudo ip link set eth1 up

Why this step is required:

- A router must have an IP address on each connected network.

- Without IPs, the router cannot perform routing, ARP, or reply to pings.

- These IPs act as the default gateway for devices in those networks.

This created two subnets:

| Network | Router IP | Used by |
| --- | --- | --- |
| 36.0.0.0/29 | 36.0.0.1 | Attacker |
| 136.0.0.0/29 | 136.0.0.1 | SecureGate |

3️⃣ Enabling IP forwarding (Routing)

Temporary:

echo 1 | sudo tee /proc/sys/net/ipv4/ip_forward

Permanent:

echo "net.ipv4.ip_forward=1" | sudo tee -a /etc/sysctl.conf

sudo sysctl -p

Why:

By default, Kali Linux behaves like a host, not a router.

To function as a router, Linux must forward packets between interfaces.

IP forwarding ON = Linux becomes a router.

Without this, packets would:

- Enter eth0

- Drop inside the router

- Never reach eth1 or vice versa.

This is the MOST important step.

4️⃣ Making IP addresses permanent

Editing:

sudo nano /etc/network/interfaces

Adding:

auto eth0

iface eth0 inet static

address 36.0.0.1

netmask 255.255.255.248

auto eth1

iface eth1 inet static

address 136.0.0.1

netmask 255.255.255.248

Apply:

sudo systemctl restart networking

Why this step:

- /etc/network/interfaces stores static IPs permanently.

- Without this, the router would lose its IP configuration after reboot.

- A router must always keep its gateway addresses.

5️⃣ Testing connectivity (Validation)

From attacker:

ping 36.0.0.1

From SecureGate:

ping 136.0.0.1

Purpose:

- To verify that the router responds on both interfaces.

- To confirm correct IP assignment.

- To validate VMnet mappings (eth0 to VMnet2, eth1 to VMnet3).

This proves the router is active on both networks.

✔ Why this outer router is necessary in your topology

Your SecureGate firewall sits between:

Attacker → Outer Router → SecureGate → Inside networks

Without an outer router:

- Attacker would directly reach SecureGate, which is not realistic.

- You cannot simulate ISP behavior.

- You cannot route packets in/out properly.

- SecureGate wouldn’t receive forwarded packets to inspect.

The outer router acts as:

🔹 “ISP or upstream router”

🔹 Gateway for attacker network

🔹 Traffic distributor

🔹 Routing decision point

It is a crucial element in real-world network design and your simulated environment.

📌 Summary of All Steps (Short Version)

- Identify interfaces

- ifconfig

- Assign IPs

- ip addr add 36.0.0.1/29 dev eth0

- ip addr add 136.0.0.1/29 dev eth1

- Enable routing

- echo 1 > /proc/sys/net/ipv4/ip_forward

- Make permanent

- /etc/network/interfaces

- Test

- ping <router IP>

🎉 Your outer router is correctly configured and production-ready.

On securegate we followed theses steps:

Great — since SecureGate outer router is now working exactly as expected, here are the clean, final, step-by-step configuration instructions you can put directly into your documentation.
This includes what we did + why we did it, written clearly for a project report.

✅ SecureGate – Outer Router Configuration (Final Steps With Explanation)

(Use this in your documentation or practical record.)

🔹 Step 1 — Assign IP addresses to each network interface

SecureGate has three NICs (eth0, eth1, eth2), each connected to a different VMware network.

We configured:

- eth0 → Windows / External network → 136.0.0.0/29

- eth1 → Honeypot network → 222.0.0.0/29

- eth2 → Internal Server network → 200.0.0.0/29

Commands executed:

sudo ip addr add 136.0.0.2/29 dev eth0

sudo ip link set eth0 up

sudo ip addr add 222.0.0.1/29 dev eth1

sudo ip link set eth1 up

sudo ip addr add 200.0.0.1/29 dev eth2

sudo ip link set eth2 up

Why this step?

Each NIC must be assigned a unique IP in its subnet so SecureGate can act as a router and forward packets between networks.

🔹 Step 2 — Enable IP forwarding (turn Kali into a router)

sudo sysctl -w net.ipv4.ip_forward=1

Make it permanent:

echo "net.ipv4.ip_forward=1" | sudo tee -a /etc/sysctl.conf

sudo sysctl -p

Why this step?

Linux does not route packets by default.
This step enables SecureGate to forward traffic between attacker ↔ internal server ↔ honeypot.

🔹 Step 3 — Add Default Route on SecureGate Router

The router must know where to send traffic toward the external (Windows) network.

sudo ip route add default via 136.0.0.1 dev eth0

Why this step?

This sets Windows (136.0.0.1) as the “upstream gateway” for any traffic going outside SecureGate.

🔹 Step 4 — Configure Windows Routing to Server & Honeypot Networks

On the Windows machine, run CMD as Administrator:

route add 200.0.0.0 mask 255.255.255.248 136.0.0.2 -p

route add 222.0.0.0 mask 255.255.255.248 136.0.0.2 -p

Why this step?

Windows must know that the path to:

- 200.0.0.0/29 (Internal Server)

- 222.0.0.0/29 (Honeypot)

is through SecureGate (136.0.0.2).

Otherwise, Windows will not reply to pings or traffic from those networks.

🔹 Step 5 — Add Default Routes on Internal Server & Honeypot

On Server VM (200.0.0.2):

sudo ip route add default via 200.0.0.1

On Honeypot VM (222.0.0.2):

sudo ip route add default via 222.0.0.1

Why this step?

These VMs must send all outbound traffic back to SecureGate, otherwise packets will never reach Windows or attacker.

🔹 Optional Step 6 — Enable NAT for outbound connections

If your Server or honeypot needs internet through SecureGate:

sudo iptables -t nat -A POSTROUTING -o eth0 -j MASQUERADE

Why this step?

NAT hides internal IPs and allows multiple VMs to go out via one interface.

🎉 Final Verification Command List

On SecureGate:

ping 136.0.0.1   # Windows

ping 200.0.0.2   # Server

ping 222.0.0.2   # Honeypot

On Windows:

ping 136.0.0.2

ping 200.0.0.2

ping 222.0.0.2

On Server:

ping 200.0.0.1

ping 136.0.0.1

On Honeypot:

ping 222.0.0.1

ping 136.0.0.1

⭐ PART 4 — INTERNAL ROUTER CONFIGURATION (Already Done)

(Just summarizing for documentation)

IPs:

sudo ip addr flush dev eth0

sudo ip addr add 136.0.0.3/29 dev eth0

sudo ip link set eth0 up

sudo ip addr flush dev eth1

sudo ip addr add 200.0.0.1/29 dev eth1

sudo ip link set eth1 up

Add default route:

sudo ip route add default via 136.0.0.2

Enable forwarding:

sudo sysctl -w net.ipv4.ip_forward=1

Perfect question 👍
Below is a clean, documentation-ready explanation of what exactly we changed and why, written so you can directly paste it into your setup / implementation document.

🔧 SecureGate Network Setup – Changes & Rationale

This section explains the final changes made during SecureGate network configuration, along with the reason for each change.

1️⃣ Added a Dedicated WAN / Internet Interface (eth3)

🔹 What changed

A new network interface eth3 was introduced and configured using DHCP:

auto eth3

iface eth3 inet dhcp

🔹 Why this was required

- Earlier, SecureGate had only internal and lab networks (36.x, 100.x, 200.x, 222.x).

- There was no real Internet gateway, so traffic to public IPs (e.g., 8.8.8.8) failed.

- Using DHCP allows SecureGate to:

  - Automatically obtain an IP address

  - Automatically receive the default gateway

  - Work on any host network (Wi-Fi / LAN / mobile hotspot) without reconfiguration

🔹 Benefit

✔ SecureGate can access the Internet
✔ WAN IP can change dynamically without breaking routing
✔ Matches real-world firewall deployments

2️⃣ Enabled IP Forwarding for Packet Routing

🔹 What changed

IP forwarding was explicitly enabled:

up sysctl -w net.ipv4.ip_forward=1

🔹 Why this was required

- SecureGate acts as a gateway, not just a host.

- Linux does not forward packets by default.

- Without this:

  - Attacker → Internal Server traffic fails

  - SecureGate behaves like a dead-end system

🔹 Benefit

✔ SecureGate now forwards packets between networks
✔ Enables end-to-end communication
✔ Core requirement for firewall / gateway behavior

3️⃣ Implemented NAT (MASQUERADE) for Internet Access

🔹 What changed

Network Address Translation (NAT) was added:

up iptables -t nat -A POSTROUTING -o eth3 -j MASQUERADE

down iptables -t nat -D POSTROUTING -o eth3 -j MASQUERADE

🔹 Why this was required

- Internal networks use private IP ranges (100.x, 200.x, 222.x).

- These IPs cannot be routed on the Internet directly.

- NAT converts internal source IPs into the WAN IP dynamically.

🔹 Why MASQUERADE (not SNAT)

- WAN IP changes frequently (DHCP).

- MASQUERADE automatically adapts to IP changes.

🔹 Benefit

✔ Internal & SecureGate traffic reaches the Internet
✔ No hard-coded IP dependency
✔ Real firewall-grade NAT implementation

4️⃣ Fixed Incorrect Default Routing

🔹 What changed

The default route was shifted to eth3 (WAN) instead of internal lab networks.

Correct behavior:

- Internal destinations → routed internally

- External destinations → routed via eth3

🔹 Why this was required

Previously:

- Default route pointed to lab networks (136.x)

- Internet traffic failed with Destination Host Unreachable

Now:

- DHCP on eth3 installs the correct default gateway

- Routing follows Linux longest-prefix match logic

🔹 Benefit

✔ Internet traffic exits via WAN
✔ Internal traffic stays internal
✔ No routing conflicts

5️⃣ Added Explicit Route to Attacker Network

🔹 What changed

A static route was added:

up ip route add 36.0.0.0/29 via 100.0.0.1 dev eth0

down ip route del 36.0.0.0/29 via 100.0.0.1 dev eth0

🔹 Why this was required

- Attacker network is not directly connected to SecureGate.

- Outer Router (100.0.0.1) acts as the next hop.

- Without this route, reply packets never return to the attacker.

🔹 Benefit

✔ Attacker ↔ Internal Server communication works
✔ Symmetric routing ensured
✔ Accurate multi-hop routing simulation

6️⃣ Clearly Separated Network Zones

| Interface | Network | Purpose |
| --- | --- | --- |
| eth0 | 100.0.0.0/29 | External / Outer Router |
| eth1 | 222.0.0.0/29 | Honeypot Network |
| eth2 | 200.0.0.0/29 | Internal LAN |
| eth3 | DHCP (NAT) | Internet / WAN |

🔹 Why this matters

- Enables zone-based inspection

- Allows future firewall rules per interface

- Matches real enterprise firewall design

🧠 Final Result

After these changes:

✔ Attacker → SecureGate → Internal Server traffic works
✔ SecureGate can monitor and filter packets
✔ Internet access works dynamically
✔ Setup remains portable across different host networks
✔ Architecture mirrors real-world gateway devices